Normal and dichromatic colour discrimination measured with transient visual evoked potential

L.C.L. Silveira1,2, B.D. Gomes1, G.S. Souza1, A.R. Rodrigues1, C.A. Saito1, M. da Silva Filho1
1Dep. Fisiologia, Universidade Federal do Pará, Belém, Brazil
2Núcleo de Medicina Tropical, Universidade Federal do Pará, Belém, Brazil

It would be informative to have an electrophysiological method to study, in an objective way, the effects of mercury exposure and other neurotoxics on human colour vision performance. The purpose of the present work was to study human colour discrimination by measuring chromatic difference thresholds with visual evoked potential (VEP). Six young normal trichromats (24 ± 1 years old) and one deutan (26 years old) were tested. The stimuli consisted of sinusoidal isoluminant chromatic gratings made from chromaticity pairs located along four different colour directions centered on two reference points. Heterochromatic flicker photometry (HFP) protocol was used to obtain the isoluminance condition for every subject and for all chromaticity pairs. Spatial frequency was 2 cycles/deg. Presentation mode comprised onset (300 ms)/offset (700 ms) periods. As previously described, we found a negative deflection in the VEP which was related to the chromatic difference: as chromatic difference increased, amplitude increased and latency decreased. VEP response amplitude was plotted against distance in the CIE 1976 colour space between the grating chromaticities and fitted with a regression line. We found colour thresholds by extrapolating the fitting to null amplitude values. The thresholds were plotted in the CIE 1976 colour space as MacAdam ellipses. In normal trichromats the ellipses had small sizes, low ellipticity, and were vertically oriented. In the deutan subject, the ellipses had large size, high ellipticity, and were oriented towards the deutan copunctal locus. The VEP thresholds were similar to those obtained using grating stimuli and psychophysical procedures, however smaller than those obtained using pseudoisochromatic stimuli (Mollon-Reffin method). We concluded that transient VEP amplitude as a function of contrast can be reliably used in objective studies of chromatic discrimination performance in normal and altered human subjects.

Supported by CNPq and CAPES.